Rock Stability Methods — Verified Comparison (with sources)

All validity limits and key parameters re-checked against the primary sources. Items confirmed against clean source text are marked (verified); items relying on standard literature are flagged. Confidence and citations are given in the notes section below each table.

Scope of this thesis: the nearshore environment from the shoaling zone up to just past the breaker line. In CIRIA terms this is deep-to-shallow water, NOT very shallow water (Hs-toe < 70% Hso). The very-shallow inner-surf regime with fully broken heavy waves is outside scope, so the Van der Meer shallow-water form (Van Gent et al. 2004) is not used.

Table 1  Method comparison

| Method | Made for | Required inputs | Output | Damage criterion | Validity limits (verified) | Fit for cable berm |
| --- | --- | --- | --- | --- | --- | --- |
| Hudson (1953/59) | Slope armour, permeable-core breakwater, regular waves | Hs, KD, Δ, cotα | W50 / Dn50 | Implicit fixed (~0–5%), no scale | Slope armour, non-overtopped, permeable core (KD=4 non-breaking / KD=2 breaking; impermeable core only with KD=1); deep to shallow water but NOT very shallow (Hs-toe < 70% Hso); no strict cotα range published (KD tables ~1:1.5–1:3); no Tp; no N; no P; no current | Low – first estimate only |
| Van der Meer (1988/98) | Slope armour, random waves, deep water | Hs, Tm (sm), Δ, Dn50, cotα, P, N, Sd | Dn50 / Ns at a chosen Sd | Sd = Ae/Dn50² (start 2–3, failure ≈8–17) | Deep water (h > 3·Hs-toe); slope armour; cotα = 1.5–6; P = 0.1–0.6; N < 7500; som = 0.01–0.06; ξm = 0.7–7; Δ = 1–2.1 (higher Δ→cotα≥2); separate plunging/surging; no current. Valid in the outer/deep part of the domain, drops out as waves approach breaking | Low–med – slope, not berm/bed |
| Van Rijn (2019) | Horizontal/mild-slope bed protection, current ± waves | uc, Hs, Tp, h, Δ, slope, α(ks), r, γstr | required D50; + Nmr/Sd (Cheng 2002) for dynamic | r = 0.4–1.0 → θcr = r·0.05 (static); Cheng bed-load (dynamic) | D50 = 0.03–3 m; horizontal/mild slope; roughness h/(αD50) & Aw/(αD50) = 10–300; D50 is sieve size (D50 ≈ 1.2·Dn50); sparse field calibration for largest sizes | High – only one with combined wave+current; static & dynamic |
| Van Gent & Van der Werf (2014) | Toe of breakwater armour slope, waves | Hs, Tm-1,0, ht, Bt, tt, Dn50, Δ, cotα, roughness | Nod or Dn50 | Nod (displaced stones in a Dn50-wide strip) | ht/h = 0.10–0.30; h/Hs = 1.2–4.5; slopes 1:1.5 & 1:2; wave steepness sp = 0.015–0.04; waves only; non-overtopped; tested on fixed bed (no toe scour) | Med – berm geometry fits, but assumes armour slope above + down-rush |
| Herrera & Medina (2015) | Toe berm, very shallow + steep (m=1/10), breaking | hs, Hs0, L0p (Tp), Δ, Dn50 (std Bt=3Dn50, tt=2Dn50) | Nod (Eq. 12) + 90% CI | Nod = 0.5 start, 2.0 moderate, 4.0 failure (CIRIA 2007) | −0.15 < hs/Hs0 < 1.5 (negative = emerged berm); −0.5 < hs/Dn50 < 5.01; steepness 0.008–0.08 (Herrera Table, p.10); Ns = 0.81–3.36; m = 1/10; standard berm (Bt=3Dn50, tt=2Dn50); breaking waves only | Med–high for bedrock/steep breaking; fixed geometry, waves only |
| Deltares HaSPro / RoBeD (2023) | Loose-rock scour protection & cable-cover berms, offshore, current+waves | Shields θ (→ uc, Hs, Tp, h, Δ, D50), WTOP, MTOC, Dcable, side slope | berm geometry (Bberm/hberm, TOPC) + deformation class; external/interface/flexibility | MOBthresh = 0.95 recommended (case-specific allowed); classes 0–3; reshaping/wash-away at θ > ~0.06 | reshaping regime; MOB = θ/θcr (App A); θcr Soulsby Eq A.6 (incl. 1.2 factor); ks = 2.5·D50; perpendicular wave+current assumed; offshore-calibrated | High – only one explicitly for cable berms + 3 requirements |

Table 2  Situation → method (verified, no changes)

| Situation | Primary method | Supporting / check | Why |
| --- | --- | --- | --- |
| Bedrock, steep nearshore, breaking waves | Herrera & Medina (2015) | Van Gent (range check); Hudson (first estimate) | Only method for steep bottom + breaking; shows that gentle-slope formulas fail here. |
| Mobile sand, combined waves + current (crossing/obstacle) | Van Rijn (2019) | Deltares RoBeD (deformation + interface/flexibility) | Only method with genuine wave+current on a bed; Deltares covers winnowing/falling apron on sand. |
| Cable berm geometry & reshaping/deformation (any) | Deltares RoBeD (2023) | Van Rijn (D50 sizing) | Provides berm geometry + deformation class rather than only D50. |
| Quick first sizing / sanity check | Hudson / Van der Meer | — | Classical and fast, but without wave period or current. |

Sources & verification notes

Hudson (1953/59) — Source: CIRIA C683 Rock Manual (2007), §5.2.2.2 (Eq. 5.133–5.135) and Table 5.29 (p.579); Schiereck & Verhagen (2019), §8.3.2, Eq. 8.10.

VERIFIED against CIRIA. CIRIA gives NO strict cotα validity range for Hudson (unlike Van der Meer). Derived from regular-wave tests on permeable-core structures. KD = 4 (non-breaking) / 2 (breaking, per SPM 1984); impermeable core only with KD = 1. Per Table 5.29 Hudson is applicable for deep to shallow water but NOT very shallow (Hs-toe < 70% Hso). Your earlier 1.5–4 is not in CIRIA; use Schiereck’s 1:1.5–1:6 or drop the upper bound.

Van der Meer (1988/98) — Source: CIRIA C683 Rock Manual (2007), Table 5.24 (p.570, deep-water validity) and Table 5.29 (fields of application); Schiereck & Verhagen (2019), §8.3.2–8.3.3.

VERIFIED against CIRIA Table 5.24 (authoritative). Deep water (h > 3·Hs-toe): cotα = 1.5–6, N < 7500, som = 0.01–0.06, ξm = 0.7–7, Δ = 1–2.1 (for Δ up to 3.5 → cotα ≥ 2), P = 0.1–0.6 (P = 0.1 impermeable, 0.4 armour+filter on permeable core, 0.5 armour on permeable core, 0.6 homogeneous). Plunging/surging split. The shallow-water form (Van Gent et al. 2004, needs H2%) covers very shallow water only and is OUT OF SCOPE for this thesis.

Van Rijn (2019) — Source: van Rijn (2019), Critical movement of large rocks in currents and waves, Eq. 3 (θcr), Eq. 5 (wave+current), Table 1 (fc/fw ranges); Cheng (2002) for the dynamic case.

VERIFIED against the paper. Added vs your table: roughness ratio h/(αD50) & Aw/(αD50) = 10–300, and D50 is the sieve diameter with D50 ≈ 1.2·Dn50 (convert if inputs are nominal Dn50).

Van Gent & Van der Werf (2014) — Source: van Gent & van der Werf (2014), Rock toe stability of rubble mound breakwaters, validity statements in the paper (ht/h and h/Hs ranges, tested steepnesses).

VERIFIED against the paper. Added vs your table: wave steepness sp = 0.015–0.04 (tested), non-overtopped structure, and the tests used a fixed (non-mobile) bed so toe scour was excluded. Uses Tm-1,0 (spectral period) and Hs = Hm0.

Herrera & Medina (2015) — Source: Herrera & Medina (2015), Toe berm design for very shallow waters on steep sea bottoms, Eq. 12; validity ranges §2; damage levels after CIRIA Rock Manual (2007).

VERIFIED against the paper (pages you sent). −0.15 < hs/Hs0 < 1.5 is correct (negative lower bound = emerged berm). Damage levels per CIRIA (2007): Nod = 0.5 (start), 2.0 (moderate), 4.0 (failure). REMOVED “Nod = 1 = design” – it is not in Herrera’s cited criteria. CORRECTED steepness: Herrera’s own validity table (p.10, column “This study”) gives 0.008–0.08, not 0.02–0.07, and Ns = 0.81–3.36 (verify the exact steepness symbol s0p vs stp when you cite it). SCOPE NOTE: Herrera’s domain (very shallow, m=1/10, heavy breaking) sits at the edge of or just outside this thesis’s nearshore domain, so it will rarely be the governing method here.

Deltares HaSPro / RoBeD (2023) — Source: Deltares (2023), Handbook of Scour and Cable Protection Methods, §5.2–5.3 (RoBeD, p.73–75), Appendix A (mobility number, Eq. A.1 & A.6). Polynomial coefficients themselves are in the loose-rock analysis report Deltares (2023n), which is not in our set.

The handbook’s RoBeD section explicitly recommends MOBthresh = 0.95 for the rock-berm model, so “no fixed default” understates the source. MOB = θ/θcr; θcr from Soulsby (Eq. A.6, which keeps the 1.2 factor) and ks = 2.5·D50 for the mobility calc – note these differ from the Van Rijn (2019) θcr form used elsewhere.